HLS Assignment 3

(KANEKAL KOUSAR[FWC2022063])

Q)Repeat the experiment in assignment 2.3 by configuring the module as pipelining

3.1)

Header file

#ifndef MUL32 #define MUL32 #include <iostream> #include "hls_stream.h" using namespace std; using namespace hls; typedef long long out; struct inputs{ int A; int B; }; #endif

C++ code:

#include "mul32.h" void mul32(stream<inputs> &din,stream<out> &dout){ #pragma HLS PIPELINE inputs data=din.read(); dout.write(data.A * data.B); }

Test bench

#include "mul32.h" void mul32(stream<inputs> &din,stream<long long> &dout); int main(){ stream<inputs> indata; stream<long long> outdata; inputs in; long long out; int i; for (i=0;i<10;i++){ in.A=i+2; in.B=i; indata.write(in); mul32(indata,outdata); outdata>>out; cout<<in.A<<"X"<<in.B<<"="<< out <<endl; } return 0;}

Simulation report

INFO: [SIM 2] *************** CSIM start *************** INFO: [SIM 4] CSIM will launch GCC as the compiler. Compiling ../../../mul32_tb.cpp in debug mode Compiling ../../../mul32.cpp in debug mode Generating csim.exe 2X0=0 3X1=3 4X2=8 5X3=15 6X4=24 7X5=35 8X6=48 9X7=63 10X8=80 11X9=99 INFO: [SIM 1] CSim done with 0 errors. INFO: [SIM 3] *************** CSIM finish ***************

Synthesis report:

Co-simulation

INFO: [Common 17-206] Exiting xsim at Mon Mar 20 13:06:30 2023... INFO: [COSIM 212-316] Starting C post checking ... 2X0=0 3X1=3 4X2=8 5X3=15 6X4=24 7X5=35 8X6=48 9X7=63 10X8=80 11X9=99 INFO: [COSIM 212-1000] *** C/RTL co-simulation finished: PASS *** Finished C/RTL cosimulation.

3.2)

Header file

#ifndef MULFIX #define MULFIX #include <iostream> #include "ap_fixed.h" #include "hls_stream.h" using namespace std; using namespace hls; typedef ap_ufixed<28,4> fix28_4; typedef ap_ufixed<56,8> fix56_8; struct inputs{ fix28_4 A; fix28_4 B; }; #endif

C++ code

#include "mul.h" void mulf(stream<inputs> &din,stream<fix56_8> &dout){ #pragma HLS PIPELINE inputs data=din.read(); dout.write(data.A * data.B); }

Test bench

#include "mul.h" void mulf(stream<inputs> &din,stream<fix56_8> &dout); int main(){ stream<inputs> indata; stream<fix56_8> outdata; int i; inputs in={0,0}; fix56_8 out; for (i=0;i<10;i++){ in.A=i+0.6; in.B=i+0.5; indata.write(in); mulf(indata,outdata); outdata>>out; cout <<in.A<<"X"<<in.B<<"="<<out<< endl;}}

Simulation:

INFO: [SIM 2] *************** CSIM start *************** INFO: [SIM 4] CSIM will launch GCC as the compiler. Compiling ../../../mul_tb.cpp in debug mode Compiling ../../../mul.cpp in debug mode Generating csim.exe 0.6X0.5=0.3 1.6X1.5=2.4 2.6X2.5=6.5 3.6X3.5=12.6 4.6X4.5=20.7 5.6X5.5=30.8 6.6X6.5=42.9 7.6X7.5=57 8.6X8.5=73.1 9.6X9.5=91.2 INFO: [SIM 1] CSim done with 0 errors. INFO: [SIM 3] *************** CSIM finish ***************

Synthesis report

Co-simulation:

INFO: [Common 17-206] Exiting xsim at Mon Mar 20 14:13:10 2023... INFO: [COSIM 212-316] Starting C post checking ... 0.6X0.5=0.3 1.6X1.5=2.4 2.6X2.5=6.5 3.6X3.5=12.6 4.6X4.5=20.7 5.6X5.5=30.8 6.6X6.5=42.9 7.6X7.5=57 8.6X8.5=73.1 9.6X9.5=91.2 INFO: [COSIM 212-1000] *** C/RTL co-simulation finished: PASS *** Finished C/RTL cosimulation.

OBSERVATIONS:

Solution1:without pipelining function

Solution2:with pipelining function

(3.1)

(3.2)

- Pipelining allows code to run much faster because, ideally, it permits a new instruction to be issued every clock cycle(II) so that multiple instructions can be run simultaneously over the course of several clock cycles - it does not increase the speed with which an individual instruction can be completed, but the throughput is increased

- By applying the pipelining to function,the performance is increased

    - Initiation Interval (II) is decreased[initiation interval (II) –The number of clock cycles before the function can accept new data]or the throughput is increased

      - Form solution1 to solution2  the II is decreased from 2 to 1

    - The number of resources gets reduced

      - Number of LUT’S reduced

| Assignment |  | Resources |  |  |
| --- | --- | --- | --- | --- |
|  |  | LUT | DSP48E | FF |
| 2.3.1 | Without pipelining | 84 | 3 | 99 |
| 3.1 | With pipelining | 79 | 3 | 99 |
| 2.3.2 | Without pipelining | 100 | 3 | 115 |
| 3.2 | With pipelining | 95 | 3 | 115 |